LVDB Interactive Fault Simulation Software (DEWA Standard Based)

1️⃣ System Overview

The system shall simulate an LV Distribution Board (LVDB) including:

- 1 × Main 4P Isolator

- 5 × ELCBs in parallel

  - ELCB1 – 40A, 100mA  feeds to (6 MCBs)

  - ELCB2 – 40A, 100 mA feeds to (3 MCBs)

  - ELCB3 – 40A, 30mA feeds to (3 MCBs)

  - ELCB4 – 40A, 100mA  feeds to (UVR + Timer Contactor + 3 MCBs)

  - ELCB5 – 40A, 1000mA feeds to  (UVR + Timer Contactor + 3 MCBs)

The simulator should be fully interactive and touchscreen operated.

2️⃣ Program Operation Sequence (Core Logic)

🔹 Step 1 – Fault Selection Screen

When the program starts:

Display message:

“Kindly Select the Faulty Circuit”

User must select:

- Anyone MCB from:

  - ELCB1 (MCB1–MCB6)

  - ELCB2 (MCB7–MCB9)

  - ELCB3 (MCB10–MCB12)

  - ELCB4 +UVR+TIMER (MCB13–MCB15)

  - ELCB5+UVR+TIMER (MCB16–MCB18)

OR

Randomized blind test (faulty MCCB to be select by system Multiple circuits but from different ELCBs).

OR

If there is no faulty circuit simulated all MCB & ELCB work fine(to be in on position).

Once selected → System loads digital LVDB screen.

Screen Before selecting faulty cicuit                                             Screen after selecting faulty cicuit

🔹 Step 2 – Main Isolator Operation

- User touches Main Isolator knob.

- Knob animation moves to UP position.

- Indicator lamp turns GREEN.

- System becomes energized.

If isolator is OFF → No other device can be switched ON.

🔹 Step 3 – ELCB Operation

For ELCB-1 to ELCB-3.

User switches ON ELCBs one by one:

- Knob moves UP.

- Indicator changes to GREEN.

- Downstream MCBs become enabled.

If ELCB not ON → Related MCBs cannot be switched ON.

For ELCB-4 to ELCB-5.

User switches ON ELCBs one by one:

- Knob moves UP.

- Indicator changes to GREEN.

- UVR timer starts after ELCB switching ON  with 2 indications one red & one orange. After 10 seconds. Orange indication becomes Green &  contractor energized ,

- Indicator changes to GREEN for contractor

- Downstream MCBs become enabled.

If ELCB not ON → Related UVR Timer+ contractor cannot be switched ON.

🔹 Step 4 – MCB Operation (Sequential Switching)

User switches ON MCBs one by one.

For MCB from MCB-1 to MCB-12:

- Knob moves UP.

- Indicator turns GREEN.

- Load indicator becomes active.

For MCB from MCB-13 to MCB-18:

- Knob moves UP .

- Indicator turns GREEN.

- Load indicator becomes active

3️⃣ Fault Trigger Logic (Very Important)

When the user reaches the pre-selected faulty MCB:

System must:

1️⃣ Trip its respective ELCB
2️⃣ Change ELCB indicators to RED
3️⃣ Knobs move to DOWN position
4️⃣ Display message:

“Earth Leakage Fault Detected on [MCB Name]”

Example:
If fault selected = ELCB2 → MCB2
Then:

- MCB2 trips

- ELCB2 trips

- Other ELCBs remain ON

4️⃣ Protection Behavior Logic

| Fault Location | Result |
| --- | --- |
| Fault under ELCB1 | Only ELCB1 trips |
| Fault under ELCB2 | Only ELCB2 trips |
| Fault under ELCB3 | Only ELCB3 trips |
| Main isolator OFF | Entire DB de-energized |

Selective tripping must be implemented.

5️⃣ Visual & Interface Requirements

The LVDB screen must include:

✅ Single Line Diagram layout
✅ Realistic toggle knob animation
✅ Color Indicators:

- Green = ON

- Red = Trip

- Grey = OFF

✅ Event message window
✅ Reset button
✅ Back to fault selection screen
✅ Alarm sound (optional)

6️⃣ Reset Logic

After fault trip:

User must:

- Switch OFF all MCB of respective ELCB.

- Switch OFF tripped ELCB(Already in tripped position).

- Press RESET button

- Then restart procedure

System should not auto-reset.

7️⃣ Optional Advanced Features (Recommended)

- Leakage current display (mA value)

- Fault history log

- Instructor mode (hidden fault selection)

- Timer contactor delay simulation

- UVR drop during voltage dip simulation